NIBE F1145/F1245  Version History

Koneen toimitusversio oli 1725.

Version: 3105R3

Asennettu meidän koneeseen 2013-05-13.

Julkaistu kaiketi jo 2012 puolella.

Ei tietoa muutoksista.

Version: 5539R7

Latest revision of this document: 2014-02-25

Release date: 2014-01-31

Asennus 27.2.2014

o Added setup of time zone in menu 4.4 ”time & date”

o Added menu 4.1.3”Internet” and submenus for setup of NIBE Uplink.

o Added NIBE Smart Price Adaption in menu 4.1.6. (only apply to heat pumps sold in Sweden)

Circulation pumps

o Added automatic speed adaption of brine circulation pump.

o Added automatic speed adaption of heating circulation pump.

o Added menu 5.1.14 for settings of system when activating automatic heating circulation pumps in menu 5.1.11.

Accessories

o F1145/F1245 support for ACS45 (4-pipe active and passive cooling)

o Possible to control up to two pools.

o Added setting for “Prioritize addition” when running accessory “Shunt controlled addition” in menu 5.3.2

o Additional functionality in FLM control/setting with new cooling feature.

o Modbus address now possible to change in menu 5.3.11. Supported from MODBUS40 version 8 and up.

Master/slave

o Added master/slave functionality for combining up 9pcs F1145/F1245 and F1345.

o Added master/slave menus in 5.2

o Menu “accessories” moved to 5.2.4

o External addition changed from absolute value to “start diff addition” setting in 4.9.3

o New alarm numbers, #301-308, added to cover master/slave functionality.

o When running F1145 in master slave possible to configure for “own” hot water or “common” hot water.

o Hot water control improved and now controlled based on setting ‘Step diff hot water”: x K” in (menu 5.1.1)

Menus

o Added tabs to cover new functions and features

o Added Qr-code in service menu 3.1

o Energy metering now showing delivered energy per demand. NOTE: Old values will be set as heat.

o General update and improvement of translations

o Moved cooling setting for ECS 2-4 from 1.9.5 to 5.3.3.2 and 5.3.3.4

Functions

o SG ready functionality added

o Aux inlets now configurable with additional features

 External alarm (NC or NO), info alarm 995”alarm from external source”.

 Return sensor temp(BT71)

 SG Ready A

 SG Ready B

 +Adjust

 Flow temperature sensor cooling (BT25)

 Possible to choose between Activate fan speed (NC) or Activate fan speed (NO). Choices apply to installed FLM

o Faulty sensor connected to AUX gives alarm

o Upon start of heat pump, circulation pumps perform sensor calibration on both heating and brine side.

Changes

o Default setup for HWC changed where circulation pumps now runs 100% of the time

o Improved setup range for stop addition in menu 4.9.2 from -20C to -25C.

 General bug fixes and improvements

Version: 5539R8

Asennus 13.3.2014

Version: 6629R3
Release date: 2015-01-13
Changes
• Upon firmware upgrade country must be chosen and is indicated by info alarm (900) until selected.
• Time zone is set automatically when you change country in ”country selection”.
• Other minor bug fixes and improvements.

Version:        7113R3
Release date: 2015-11-06

Asennus: 28.11.2015

Added
• Support for flow meter. 
• Possibility to choose ”Sum alarm” on AUX-out. 
• Accessory shunt controlled brine.
• Support for Smart home function. 
Changes
• Smaller adjustments for viewing and managing menues. 
• Improved hot water charging.
• 24h after the country selection in menu 5.12 is made; the menu is locked and it is no longer possible to change. 
• Improved finnish translation. 
• It is now possible to choose speed ”auto” and speed in standby in menu 5.1.9 (brine pump speed) 
• FLM: 
- Accessory menu FLM 5.3. Can now also set the pump speed. 
- Improved control of FLM cooling; is now controlled by AZ1-BT50 sensor compared to previous 
heat/ cool sensor BT74 (When upgrade to v7113R3, if FLM cooling is controlled by 
cool/heat sensor BT74 is must be changed to AZ1-BT50 in AUX-in). 
• Improved system stability
• Improved management of alarm functions
• Other minor bug fixes and improvements

Version: 7113R4
Release date: 2016-02-03

Asennus:  12.3.2016

Changes
• Improved smart home functionality
• Improved control of circulation pump in FLM
• Now supports brine-pump Grundfos UPMXL
• Other minor bug fixes and improvements

Version: 7113R5

Release date: 9/2/2016

Asennus:  20.9.2016

Added

• Smart Price Adaption for Estonia Changes

• Improved function  of brine pump speed

• Other minor bugfixes and improvements

Version: 7113R6

Release date:  9/30/2016

Added

• Support added for new type of memory circuit for the display unit.

This Update affects only upcoming displays with changed hardware.

Version: 7740R2

Release date: 11/3/2016

Asennus:  6.11.2016

Added

• Vacation can now be set in AUX-out (menu 5,4)

• Icelandic  language added

• Now support accessory SES and OPT

• Now supports sensor BT57 and BT58 for the accessory ground water pump (AXC40) with the possibility to activate min out-temperature

• ”One time increase” is now optional in menu 2.1 (Temporary lux). When activating a temporary increase will start and stop when the need is met.

• Now supports accessory EME10

• It is now possible to choose EMK for slaves

• Now support accessory ERS10

• In F1145*  it is now possible to choose hotwater charge method  ”target temp” in menu  5.1.1

Changes

• Auto settings;  start addition is now default 5 degrees

• Minor bugfixes and improvements

• Minor Adjustements of menus and texts

• Improved network stability

• Improved solar cooling function on Solar 40

• If country selection Belgium; the effect is now limited to 1electrical step. (0.5kW/1kW/1.33kW  depending on the immersion heater)

Version: 7740R4

Release date: 11/30/2016

Asennus:  5.12.2016

Changes

• Solves a problem with the firmware v7740R2 that could not to be shown in menu 7.1 if update of an older F11/1245 with a display without Internet connection

Version: 7968R2
Release date: 10.4.2017

Added
· ”Åland” is added in the country selection menu
· ERS: Possible to choose bypass function without BT50 and blocked heat
· Soft output (AUX) now also shows status in the service info 3.1,it used to only show function
· SMS40 now also sends SMS when a filter alarm occurs
· It is now possible to choose the amount of pulses/kWh or energy/puls for external energy meter in
menu 5.3.
· AXC40: Possible to regulate step control addition with 0-10V signal
· Improved function for the accessory EMK.

Changes
· It is now possible to choose fuse size up to 400A.
· Smaller adjustments of menues and menu texts.
· Improved automatic resetting of the alarm "low KB" with HPAC accessory

Versio: 8224
Julkaisupäivä: 19.9.2017 

Added
• Supports the accessory EME20
• ”Activate economy” can now be selected on the AUX-in (Menu 5.4)
• ”Smart control” can now be selected in menu 2.2
• Smart Price Adaption is now supported if country selection Norway
• Electric price region ”AWAT1” can be selected in menu 4.1.6 (Smart Price Adaption)

Changes
• Improved control of Smart homefunction
• MODBUS40: It is now possible to read 32-bites register as two separete 16-bites register and the two following 16-bites at the same time
• AXC40: It is now possible to control the ground water pump with 0-10Vsignal
• Max flow temperature can now be set up to 80 °C if external flow temperature sensor BT25 is connected
• Smaller adjustements of menus and menu texts.

Versio: 8538R2

Julkaisupäivä: 14.3.2018

Asennus 21.8.2018

Added

- Now supports 4 units of the accessory ERS

- Value from external energy meter connected to slave, is now showing in master

- Country selection Australia and New Zeeland added

- Stove guard function added for ERS

- Improved integration with Smart Price Adaption toward NIBE Uplink API

- Now supports 4 units of the accessory HTS

- It is now possible to choose night cooling, separately for each FLM in menu 5.3.1

- External adjustment cooling can now be set in menu 1.9.2

Changes

- Selected brine shunt control could earlier be deactivated at a restart of the heat pump, this is now corrected.

- Time zone Istanbul is now corrected and is always delayed with 3h compared to UTC

- Improved control of ground water pump

- AXC40: Now possible to control shunt control addition with 0-10V signal

- Solves a problem that made the choice ”intermittent” not showing in menu 5.1.10 if heat pump was selected as slave.

- EME10 is now regulated on power instead of current. New setting in menu 4.1.10 which indicates if EME10 is on single or three phase.

- Solved a problem with displayed measured flow with the accessory EMK in Uplink. Previously measured value could be lost at low flows.

- Removed a term for country selection Belgium: Max internal addition was 1kW.

- EME20 now supports up to 12 inverters

- Min supply for the accessory HPAC can now be set down to -5, earlier +5 grader in menu 1.9.3.2.

- Load monitor: Solved a problem with the alarm 182 could indicate when a new internal electric step were going to step in.

- Updates of texts and translations

- Improved control of the accessory HPAC at operation mode active cooling and run priority is hot water or pool.

- Solves a problem that could make the product to restart if the activated vacation period was longer than 127 days.

- Improved control of accessory ERS

- Improved control of heat (GP1) and brine pump (GP2)

Version: 9172R2
Publication date:18.4.2019

Ei asennettu

Added

· Is now possible to choose “heat medium return (BT71)” on AUX-in (Menu 5.4)

· Alarm for pressure switch climate system added. Activates through AUX-input. Alarm number 165

“Low pressure climate system”. The alarm is a redlight alarm without control action but lights the

red diode and can give a notification through Uplink or SMS.

· Added a solution to problems scrolling in the menus. Prevents the display pointer from going back

and forth. This eliminates the need to replace the display unit due to this problem.

· Added possibility to set minimum allowed speed in heating for heating medium pump

· Now supports the accessory external energy

· Now supports master/slave function with Fxx55 models#NIBE * Extra climate system 2 - 8: It is now possible to block the shunt on AA5-X2:13-14. When there is a connec- tion the shunt closes

· EME20: It is now possible to choose “priority household electricity” in menu 4.1.11 (Solar

electricity) if external energy meter is connected.

· New setting in menu 5.1.9 (brine pump speed), “Speed extern. contr (AUX)” by activating brine

pump on AUX-in.

Changes

· EME20: Improved communication with the accessory.

· Improved control of heating medium pump GP1

· ACS (active/passive cooling): Improved control

· Improved display of “my icons”. Prevents that part of the outdoor temperature is hidden when the

alumi- num door is closed.

· Minor adjustments of menues and texts.

· Solved incorrect names on climate system 2-8 in service info

· Improved control of the accessory HTS

· EME10: In the automatic search of accessories, EME10 could incorrectly be activated even though it was not installed.

Version: 9240R3

Release date: 11/11/2020

Added

• Popup menu is now requesting a restart of the machine if selected protocol is changed in menu

5.2.1

Changes

• Improved control of ground water pump

• Improved control of brine pump in passive cooling

• Improved control of heating mediump pump (GP1).

Version: 9478R1

Release date: 7/9/2021

Added

• It is now possible to select Charging method “target temp” for hot water charging with several

compressors

• Support for new display hardware.

Changes

• Updated texts and translations

• Fixed an issue with heating medium pump control in active cooling

• EME alarm 504 (The inverter has a message) changed from red light alarm to info alarm. Does no

longer appear in the alarm log

• When a demand for POOL 2 stops, a “Top up” of hotwater i performed.

• ECS shunts now close during hot water charging and pool to prevent water from circulating

backwards in climate system 1, Does not apply if sensor BT25 is installed

• Default is now 7 days for time between periodic increase of hot water

• Fixed an issue where NV10 alarms did not block slaves in multi-operation

• Fixed an issue where Alarm 371 “low groundwater temperature” did not block slaves in multi-

operation

• The serial number of products manufactured on day 366 (leap year) is now accepted by the software