NIBE F1145/F1245  Version History

Koneen toimitusversio oli 1725.

Version: 3105R3

Asennettu meidän koneeseen 2013-05-13.

Julkaistu kaiketi jo 2012 puolella.

Ei tietoa muutoksista.

Version: 5539R7

Latest revision of this document: 2014-02-25

Release date: 2014-01-31

Asennus 27.2.2014

o Added setup of time zone in menu 4.4 ”time & date”

o Added menu 4.1.3”Internet” and submenus for setup of NIBE Uplink.

o Added NIBE Smart Price Adaption in menu 4.1.6. (only apply to heat pumps sold in Sweden)

Circulation pumps

o Added automatic speed adaption of brine circulation pump.

o Added automatic speed adaption of heating circulation pump.

o Added menu 5.1.14 for settings of system when activating automatic heating circulation pumps in menu 5.1.11.

Accessories

o F1145/F1245 support for ACS45 (4-pipe active and passive cooling)

o Possible to control up to two pools.

o Added setting for “Prioritize addition” when running accessory “Shunt controlled addition” in menu 5.3.2

o Additional functionality in FLM control/setting with new cooling feature.

o Modbus address now possible to change in menu 5.3.11. Supported from MODBUS40 version 8 and up.

Master/slave

o Added master/slave functionality for combining up 9pcs F1145/F1245 and F1345.

o Added master/slave menus in 5.2

o Menu “accessories” moved to 5.2.4

o External addition changed from absolute value to “start diff addition” setting in 4.9.3

o New alarm numbers, #301-308, added to cover master/slave functionality.

o When running F1145 in master slave possible to configure for “own” hot water or “common” hot water.

o Hot water control improved and now controlled based on setting ‘Step diff hot water”: x K” in (menu 5.1.1)

Menus

o Added tabs to cover new functions and features

o Added Qr-code in service menu 3.1

o Energy metering now showing delivered energy per demand. NOTE: Old values will be set as heat.

o General update and improvement of translations

o Moved cooling setting for ECS 2-4 from 1.9.5 to 5.3.3.2 and 5.3.3.4

Functions

o SG ready functionality added

o Aux inlets now configurable with additional features

 External alarm (NC or NO), info alarm 995”alarm from external source”.

 Return sensor temp(BT71)

 SG Ready A

 SG Ready B

 +Adjust

 Flow temperature sensor cooling (BT25)

 Possible to choose between Activate fan speed (NC) or Activate fan speed (NO). Choices apply to installed FLM

o Faulty sensor connected to AUX gives alarm

o Upon start of heat pump, circulation pumps perform sensor calibration on both heating and brine side.

Changes

o Default setup for HWC changed where circulation pumps now runs 100% of the time

o Improved setup range for stop addition in menu 4.9.2 from -20C to -25C.

 General bug fixes and improvements

Version: 5539R8

Asennus 13.3.2014

Version: 6629R3
Release date: 2015-01-13
Changes
• Upon firmware upgrade country must be chosen and is indicated by info alarm (900) until selected.
• Time zone is set automatically when you change country in ”country selection”.
• Other minor bug fixes and improvements.

Version:        7113R3
Release date: 2015-11-06

Asennus: 28.11.2015

Added
• Support for flow meter. 
• Possibility to choose ”Sum alarm” on AUX-out. 
• Accessory shunt controlled brine.
• Support for Smart home function. 
Changes
• Smaller adjustments for viewing and managing menues. 
• Improved hot water charging.
• 24h after the country selection in menu 5.12 is made; the menu is locked and it is no longer possible to change. 
• Improved finnish translation. 
• It is now possible to choose speed ”auto” and speed in standby in menu 5.1.9 (brine pump speed) 
• FLM: 
- Accessory menu FLM 5.3. Can now also set the pump speed. 
- Improved control of FLM cooling; is now controlled by AZ1-BT50 sensor compared to previous 
heat/ cool sensor BT74 (When upgrade to v7113R3, if FLM cooling is controlled by 
cool/heat sensor BT74 is must be changed to AZ1-BT50 in AUX-in). 
• Improved system stability
• Improved management of alarm functions
• Other minor bug fixes and improvements

Version: 7113R4
Release date: 2016-02-03

Asennus:  12.3.2016

Changes
• Improved smart home functionality
• Improved control of circulation pump in FLM
• Now supports brine-pump Grundfos UPMXL
• Other minor bug fixes and improvements

Version: 7113R5

Release date: 9/2/2016

Asennus:  20.9.2016

Added

• Smart Price Adaption for Estonia Changes

• Improved function  of brine pump speed

• Other minor bugfixes and improvements

Version: 7113R6

Release date:  9/30/2016

Added

• Support added for new type of memory circuit for the display unit.

This Update affects only upcoming displays with changed hardware.

Version: 7740R2

Release date: 11/3/2016

Asennus:  6.11.2016

Added

• Vacation can now be set in AUX-out (menu 5,4)

• Icelandic  language added

• Now support accessory SES and OPT

• Now supports sensor BT57 and BT58 for the accessory ground water pump (AXC40) with the possibility to activate min out-temperature

• ”One time increase” is now optional in menu 2.1 (Temporary lux). When activating a temporary increase will start and stop when the need is met.

• Now supports accessory EME10

• It is now possible to choose EMK for slaves

• Now support accessory ERS10

• In F1145*  it is now possible to choose hotwater charge method  ”target temp” in menu  5.1.1

Changes

• Auto settings;  start addition is now default 5 degrees

• Minor bugfixes and improvements

• Minor Adjustements of menus and texts

• Improved network stability

• Improved solar cooling function on Solar 40

• If country selection Belgium; the effect is now limited to 1electrical step. (0.5kW/1kW/1.33kW  depending on the immersion heater)

Version: 7740R4

Release date: 11/30/2016

Asennus:  5.12.2016

Changes

• Solves a problem with the firmware v7740R2 that could not to be shown in menu 7.1 if update of an older F11/1245 with a display without Internet connection

Version: 7968R2
Release date: 10.4.2017

Added
· ”Åland” is added in the country selection menu
· ERS: Possible to choose bypass function without BT50 and blocked heat
· Soft output (AUX) now also shows status in the service info 3.1,it used to only show function
· SMS40 now also sends SMS when a filter alarm occurs
· It is now possible to choose the amount of pulses/kWh or energy/puls for external energy meter in
menu 5.3.
· AXC40: Possible to regulate step control addition with 0-10V signal
· Improved function for the accessory EMK.

Changes
· It is now possible to choose fuse size up to 400A.
· Smaller adjustments of menues and menu texts.
· Improved automatic resetting of the alarm "low KB" with HPAC accessory

Versio: 8224
Julkaisupäivä: 19.9.2017 

Added
• Supports the accessory EME20
• ”Activate economy” can now be selected on the AUX-in (Menu 5.4)
• ”Smart control” can now be selected in menu 2.2
• Smart Price Adaption is now supported if country selection Norway
• Electric price region ”AWAT1” can be selected in menu 4.1.6 (Smart Price Adaption)

Changes
• Improved control of Smart homefunction
• MODBUS40: It is now possible to read 32-bites register as two separete 16-bites register and the two following 16-bites at the same time
• AXC40: It is now possible to control the ground water pump with 0-10Vsignal
• Max flow temperature can now be set up to 80 °C if external flow temperature sensor BT25 is connected
• Smaller adjustements of menus and menu texts.

Versio: 8538R2

Julkaisupäivä: 14.3.2018

Asennus 21.8.2018

Added

- Now supports 4 units of the accessory ERS

- Value from external energy meter connected to slave, is now showing in master

- Country selection Australia and New Zeeland added

- Stove guard function added for ERS

- Improved integration with Smart Price Adaption toward NIBE Uplink API

- Now supports 4 units of the accessory HTS

- It is now possible to choose night cooling, separately for each FLM in menu 5.3.1

- External adjustment cooling can now be set in menu 1.9.2

Changes

- Selected brine shunt control could earlier be deactivated at a restart of the heat pump, this is now corrected.

- Time zone Istanbul is now corrected and is always delayed with 3h compared to UTC

- Improved control of ground water pump

- AXC40: Now possible to control shunt control addition with 0-10V signal

- Solves a problem that made the choice ”intermittent” not showing in menu 5.1.10 if heat pump was selected as slave.

- EME10 is now regulated on power instead of current. New setting in menu 4.1.10 which indicates if EME10 is on single or three phase.

- Solved a problem with displayed measured flow with the accessory EMK in Uplink. Previously measured value could be lost at low flows.

- Removed a term for country selection Belgium: Max internal addition was 1kW.

- EME20 now supports up to 12 inverters

- Min supply for the accessory HPAC can now be set down to -5, earlier +5 grader in menu 1.9.3.2.

- Load monitor: Solved a problem with the alarm 182 could indicate when a new internal electric step were going to step in.

- Updates of texts and translations

- Improved control of the accessory HPAC at operation mode active cooling and run priority is hot water or pool.

- Solves a problem that could make the product to restart if the activated vacation period was longer than 127 days.

- Improved control of accessory ERS

- Improved control of heat (GP1) and brine pump (GP2)

Version: 9172R2
Publication date:18.4.2019

Ei asennettu

Added

· Is now possible to choose “heat medium return (BT71)” on AUX-in (Menu 5.4)

· Alarm for pressure switch climate system added. Activates through AUX-input. Alarm number 165

“Low pressure climate system”. The alarm is a redlight alarm without control action but lights the

red diode and can give a notification through Uplink or SMS.

· Added a solution to problems scrolling in the menus. Prevents the display pointer from going back

and forth. This eliminates the need to replace the display unit due to this problem.

· Added possibility to set minimum allowed speed in heating for heating medium pump

· Now supports the accessory external energy

· Now supports master/slave function with Fxx55 models#NIBE * Extra climate system 2 - 8: It is now possible to block the shunt on AA5-X2:13-14. When there is a connec- tion the shunt closes

· EME20: It is now possible to choose “priority household electricity” in menu 4.1.11 (Solar

electricity) if external energy meter is connected.

· New setting in menu 5.1.9 (brine pump speed), “Speed extern. contr (AUX)” by activating brine

pump on AUX-in.

Changes

· EME20: Improved communication with the accessory.

· Improved control of heating medium pump GP1

· ACS (active/passive cooling): Improved control

· Improved display of “my icons”. Prevents that part of the outdoor temperature is hidden when the

alumi- num door is closed.

· Minor adjustments of menues and texts.

· Solved incorrect names on climate system 2-8 in service info

· Improved control of the accessory HTS

· EME10: In the automatic search of accessories, EME10 could incorrectly be activated even though it was not installed.

Version: 9240R3

Release date: 11/11/2020

Added

• Popup menu is now requesting a restart of the machine if selected protocol is changed in menu

5.2.1

Changes

• Improved control of ground water pump

• Improved control of brine pump in passive cooling

• Improved control of heating mediump pump (GP1).

Version: 9478R1

Release date: 7/9/2021

Asennettu: 10.8.2022

Added

• It is now possible to select Charging method “target temp” for hot water charging with several

compressors

• Support for new display hardware.

Changes

• Updated texts and translations

• Fixed an issue with heating medium pump control in active cooling

• EME alarm 504 (The inverter has a message) changed from red light alarm to info alarm. Does no

longer appear in the alarm log

• When a demand for POOL 2 stops, a “Top up” of hotwater i performed.

• ECS shunts now close during hot water charging and pool to prevent water from circulating

backwards in climate system 1, Does not apply if sensor BT25 is installed

• Default is now 7 days for time between periodic increase of hot water

• Fixed an issue where NV10 alarms did not block slaves in multi-operation

• Fixed an issue where Alarm 371 “low groundwater temperature” did not block slaves in multi-

operation

• The serial number of products manufactured on day 366 (leap year) is now accepted by the software

Version: 9699R4

Release date: 4.6.2024

Added

• AUX output with flow protection cooling indication. Possible to select NO or NC

• AUX input, “Flow guard brine”

• AUX input to allow heating/cooling even if operation mode not allow it

• Hot water out (BT70) and Hot water circulation (BT82) sensors for Hot water circulation sys- tems

on AUX-in

• New text translations

• Support for Portuguese language

• Pump exercise on GP10

Changes

• Connects to myUplink instead of NIBE Uplink. Connected systems are migrated. The owner needs to register an myUplink account with the email address used for their previous NIBE Uplink account to gain access to updated online service

• Changed default DOT temperature for Netherland and Belgium to -10 degrees

• Smart price Adaptation: Updated function for better handling of negative electricity prices

• Fixed alarm handling of BT5/BT7 sensor alarms when sensors are attached to AUX input

• Fixed internal addition being blocked if 0.0kW is selected in menu 5.1.12

• Minor adjustment of menu and menu texts

Version: 9699R5

Release date: 3.7.2024

Changes

• Fixed issue with the unit not connecting to myUplink after update

Version: 9699R7

Release date: 10.9.2024

Asennettu: 18.11.2024

Added

• New Nord pool / Epex Spot zones for Smart Price Adaption (GER, DE-LU, PL, FR, AT)

• The TSS20 accessory is now supported

Changes

• Fixed issue with menu 4.1 not visible on myUplink even when SPA is available

• Fixed issue with room setpoint and paralell offset not visible on tile

• Fxx45-6; Fixed a problem that caused HP alarms during hot water production

• Updated functionality against myUplink